Machine Learning enabled Life Cycle Assessment for Early-stage Sustainable Process Design

Kirti M. Yenkiea*, Emmanuel A. Aboagyea, Austin L. Lehra, John Pazika, Jared Longoa, and Robert P. Hesketha

aDepartment of Chemical Engineering, Rowan University, 201 Mullica Hill Road, Glassboro, New Jersey - 08028, USA

*yenkie@rowan.edu

Abstract

Holistic assessment during early-stage process design is important for developing environmentally sustainable processes. Life cycle assessment (LCA) is a systematic analysis of potential environmental impacts of raw materials, products, or services during their entire life cycle. LCA requires life cycle inventory (LCI) data for all chemicals and technologies involved within a process, which is not readily available for new alternative chemicals, solvents, and technologies. To this end, this paper presents an innovative method involving machine learning (ML) approaches whereby LCIs can be readily predicted with minimal data collected from the exploratory phase of process design. This dataset consists of physicochemical, and structural properties of known chemicals as well as their LCI labels from databases such as the Ecoinvent. ML regression algorithms such as Artificial Neural Networks (ANN) are used to develop models/correlations for LCI label predictions based on the chemical and structural properties. To illustrate the applicability of these LCI prediction models, a case study on valorization of red wine pomace (acetone basis) through solvent extraction is presented consisting of the complete cradle-to-cradle LCA. The LCIs from ML provide the cradle-to-gate numbers, the red wine pomace valorization process simulation provides the materials and energy consumption in the gate-to-gate phase, and the gate-to-cradle phase is incorporated via the solvent recovery through distillation.

Keywords: Environmental Impacts, Life Cycle Inventories, cradle-to-cradle LCA

Introduction

The global chemical market is projected to double between 2017 to 2030, which brings associated concerns for the environment as the chemical waste and releases are bound to increase at an equal or higher rate (US EPA, 2013). A common way to quantify the amount of waste produced is an E-factor (Sheldon 2007), as given by Equation 1. The E-factor values can range between 0.01-100 depending on the type of industry and its products. The pharmaceutical industry is one of the highest waste contributors with E-factor in the range of 25-100.

| E-factor = Total mass waste produced Total mass product produced | (1) |
| --- | --- |

Furthermore, the growing chemical market is an indication of the development of novel chemicals and processes. New chemicals often lack data needed to accurately quantify their potential environmental impacts, requiring experimentation or computationally intensive simulations to acquire them. These expensive and time-consuming methods can discourage many small-scale production facilities from being able to consider greener or safer alternatives.

Life cycle analysis (LCA) is a systematic analysis of potential environmental impacts of raw materials, products, or services during their entire life cycle (Karka et al., 2019). In previous studies (Narciso and Martins 2020), Machine Learning (ML) has demonstrated its utility in improving energy efficiency and predicting carbon footprints of corporations. On similar lines, this work employs ML for predicting reasonably accurate Life Cycle Inventory (LCI) values for chemicals, especially, for new molecules and process technologies. This innovative method can subsequently lead to safer, more sustainable alternative chemicals and circular process design. Predicting LCI metrics for environmental indicators at the preliminary stages of design can be crucial for advancing greener and sustainable processes. Moreover, ML would allow for this LCI data to be acquired with limited time and computational resources as compared to experimental investigations and high-end molecular scale computations.

Methodology: Developing ML Algorithms for LCI Predictions

Figure 1. Overview of ML Model workflow

This section discusses the data collection and preprocessing techniques used in this work. This is followed by the two ML algorithms - Artificial Neural Networks (ANN) and eXtreme Gradient Boosting (XGBoost not discussed in this paper), ending with a discussion about the steps involved in the tuning of the hyperparameters for each model. Figure 1 presents the overall workflow, which is organized into two general steps of data preprocessing and model building.

Data Collection

- Developing a fast and reliable method for acquiring the necessary data is integral for ML model development as a large amount of data is needed to train and test the model. Models trained with larger data sets make better predictions. The model uses an automated data collection process which is divided into two main parts: (i) input feature dataset and (ii) output label dataset. Feature dataset is independent input set from which a prediction is made. Label dataset is what the ML model is trying to predict. In this work, the feature set is comprised of both thermodynamic and molecular descriptor properties of the chemicals, while the label dataset consists of four endpoint impact assessment metrics: human health (HHI), ecosystem quality (EQI), climate change (CCI), and resource utilization (RUI) impacts.

The CAS database comprises of many organic molecules because of their prevalence in chemical industry, thus, they were used in the training set. In total, 16 thermodynamic properties and 200 molecular descriptors were collected, as well as each chemical name, formula, CAS number, and SMILES structure. Priority was placed on properties that directly affected the energy costs of certain processes, such as heat of vaporization, enthalpies of formation, and boiling points. The three main python libraries used in feature extraction of the thermodynamic properties were the “chemicals”, “thermo”, and “pubchempy” packages. Each package connects to databases such as PubChem, National Institute of Standards and Technology (NIST), and other reputable and publicly available databases. To collect the molecular descriptors for our training set, the python package "rdkit" was used. RDKit is an open-source toolkit developed primarily for cheminformatics. Its core algorithms are in C++ but there are python wrappers that can be used to access the molecular descriptors, provided their SMILEs code is known. A total of 216 features were collected for 502 organic chemicals. The next step was to collect the corresponding LCI metrics for all the 502 chemicals. Multiple methods were used such as the Ecoinvent database and IMPACT 2002+ method to collect the label data - HHI, EQI, CCI, and RUI metrics. Table 1 presents few examples of the type of data extracted for this work.

- Table 1. Example of data collected for few chemicals.

| Chemical​ Name | Chemical Formula | CAS # | SMILES | Heat of vaporization (kJ/kg) | Heat Capacity (kJ/kgK) | Number of Hetero-atoms | Climate Change (kgCO2eq) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-Bromopro-pane | C3H7Br | 106-94-5 | CCCBr | 32200 | 1.09 | 1 | 4.677 |
| 1-chloro-1,1-difluoro-ethane | C2H3ClF2 | 75-68-3 | CC(F) (F)Cl | 20400 | 1.31 | 3 | 3.979 |
| 1,1,1-trichloro-ethane | C2H3Cl3 | 71-55-6 | CC(Cl) (Cl)Cl | 29900 | 1.08 | 3 | 2.085 |
| 1,1,2,2-tetrachloro-ethane | C2H2Cl4 | 79-34-5 | ClC(Cl) C(Cl)Cl | 44400 | 0.99 | 4 | 2.894 |

Figure 2 Clustering for missing data prediction

Data Clustering

Once the data was obtained, further work was done to make it optimal for the final ML model. Much of the data was missing and needed to be filled in since the ML model cannot use null values. A K-means clustering algorithm was determined to be the best way to fill in the missing data, and the following flowchart describes the general process for creating a new dataset without any missing values.

In the dataset, rows represent chemicals and columns represent the various thermodynamic properties and molecular descriptors. Columns with missing data were temporarily removed to create a smaller dataset containing all the chemicals and a subset of the properties (with no missing data). This subset was then fed into the algorithm to create different groupings of chemicals that the algorithm determined were similar. Once clustered, some chemicals appeared as outliers. These chemicals were manually reviewed for error in the data scraping process. The algorithm was then rerun with the new fixed dataset.

After looking at the graph of the accuracy of the clusters compared to the number, the elbow method was used to determine that the optimal number of clusters. Nearly identical scripts were then made for each of the different clustered datasets. For example, when looking at the script where the chemicals were clustered into four groups, the algorithm sorted each of the five hundred chemicals into one of four groups based on the properties. Within each separate dataset, the average for each column is then taken and replaces the missing data in that column. Every column in every separate data set undergoes this process, removing all empty data by replacing it with the column average in that cluster. Once this is complete, the separate datasets then recombine to create a new dataset with no missing values. The average values act as estimates for the missing data. To check the clustered values’ accuracy, the estimated density for fifteen different chemicals were chosen and compared to the known density values found in the DIPPR database. Density was chosen because it about half of the chemicals needed it to be estimated and the known densities are readily available in DIPRR to cross compare for accuracy.

Developing the ML Algorithm - ANN

- When training a ML model, the data is split into a testing and training set, and optionally a cross validation set. The training data is used to fit the model; this data is put into the program to create and adjust the variables used in the model to generate predictions. The cross-validation set is then used for the program to compare the model fitted with the training data set. The program then runs the model with the cross validation set and adjusts the weights according to how accurate the predicted outputs are to improve the model. Finally, the testing set is used to run the model and compare the predicted outputs of the testing set to the already known output values to find the accuracy of the ML model.

- Out of the two models, the ANN model for ML is described in detail. It can be used for a wide variety of applications and is easily accessible for all Python users.

Figure 3 Basic layout of neural network

The architecture for an ANN model is shown in Figure 3. There is one input layer comprised of a number of nodes. Then there are hidden layers containing several nodes, of which the user can designate the desired number of layers and nodes in each layer. The last layer is the output layer, comprised of nodes which cannot exceed the number in the input layer. Additionally, the output layer should correspond to the number of labels you are trying to predict. Every node in one layer receives an input from all the nodes from the preceding layer, processes that information, and passes the outcome to the following layer (Géron 2019).

Hyperparameter Tuning

Using the default hyperparameters in the ANN algorithm often does not produce the best predictions. Therefore, these hyperparameters must be adjusted to this specific case, referred to as hyperparameter tuning. Different algorithms use different hyperparameters, but there are some commonalities among hyperparameters across multiple algorithms. The most important hyperparameters regarding ANN models are the hidden layers, number of nodes, and step size. Hidden layers are shown in Figure 3 and act as the overall complexity and size of the model. The number of nodes in each hidden layer displays the size and complexity of each layer, and the rate that the model changes is the step size. These hyperparameters help improve the accuracy of the model and help prevent overfitting and underfitting.

Results and Discussion

Over 200 features were included in the data set, collected using the code described previously. The main benefits of narrowing down the number of features used in the model are to simplify the input data needed and to minimize the amount of data users would need to input to use the model for finding the LCI of their chemicals. Two feature selection tools were examined from the sklearn package: SequentialFeatureSelector and SelectFromModel. All statistics and accuracy numbers included in this results section use the SequentialFeatureSelector. Sklearn’s StandardScaler was used to scale the data, and the data was split into training, cross validation, and testing sets. The percentages for the split can vary, but the numbers included in this section use a 70/18/12 split, respectively.

Results from the ANN ML Model

Figure 4 Results from the ANN ML model for the 4 LCI labels

In ANN, a model was fitted using the training data and cross validation set. The hyperparameters tuned were learning rate, units, batch size, epochs, activation function, and number of layers. The loss function used was mean squared error. The LCI value of 4 labels were analyzed and shown in Figure 4, which also shows the root mean squared error and R2 values.

    - Results from Case Study: Wine Pomace Valorization

- The LCI data from the ML predictions can be used in conjunction with data from case studies in order to conduct a holistic LCA analysis. There are three major life phases within a chemical’s lifespan: the production phase, the use phase, and the end-of-life phase. While both the use phase and the end-of-life phase can be modeled using simulation tools such as ASPEN/SuperPro, the LCI for the production phase cannot be easily modeled using traditional methods. As a result, the ML algorithm predicts the cradle-to-gate LCI data, which represents the production phase. By employing a case study, it is possible to incorporate all a chemical’s life phases to produce a complete picture of the impact it has throughout its entire lifetime. Selected case study (Vega et al. 2021) describes a process involving recovery of polyphenols from wine pomace. This process involved grinding of wine pomace followed by extraction of polyphenols using a solvent comprised of 67%wt acetone. Following the extraction, spent pomace was pressed and desolventized, with the vapors then entering a condenser. Meanwhile, the liquid stream from the presser entered a distillation column, where acetone was separated from polyphenols dissolved in water and recycled. The water-polyphenol mixture from the distillation column then entered both a nano filter and a spray dryer, with dry polyphenols serving as the final product. It is estimated that 2% of acetone is lost in each cycle, requiring make-up. The overall cradle-to-cradle results are shown in Figure 5.

Figure 5 Results for cradle-to-cradle LCA for wine pomace valorization case study

Conclusions

- In this study, the ANN-ML algorithm, was used to create a program that predicts the environmental impacts of chemicals in the areas of human health, ecosystem quality, global warming potential, and resource utilization with the goal of creating a tool that can be used to predict the effects of novel chemicals and processes. The methodology for data collection, preprocessing, model development and hyperparameter tuning are discussed. Sample results from the ANN model for 4 LCI labels are presented along with a validation case study for wine pomace valorization using acetone as the solvent-basis. This case study showed that ML model predictions can be used to substitute unknown data for cradle-to-gate LCI. Hence, ML was able to fill in the gaps and predict LCI information, allowing for a complete cradle-to-cradle analysis.

References

A. Sheldon, 2007. “The E Factor: Fifteen Years On.” Green Chemistry 9 (12): 1273–83.

Croxatto Vega, Giovanna, Joshua Sohn, Juliën Voogt, Morten Birkved, Stig Irving Olsen, and Anna Ekman Nilsson. 2021. Combining Techno-Economic and Life Cycle Assessment – a Case Study of Red Wine Pomace. Resources, Conservation and Recycling 167 (April): 105318.

Géron, Aurélien. 2019. Hands-On Machine Learning with Scikit-Learn, Keras, and TensorFlow: 2nd edition. Beijing China ; Sebastopol, CA: O’Reilly Media.

Karka, Paraskevi, Stavros Papadokonstantakis, and Antonis Kokossis. 2019. Predictive LCA - a Systems Approach to Integrate LCA Decisions Ahead of Design.” Computer Aided Chemical Engineering, 46:97–102. 29 European Symposium on Computer Aided Process Engineering.

Narciso, Diogo A.C., and F.G. Martins. 2020. Application of Machine Learning Tools for Energy Efficiency in Industry: A Review. Energy Reports 6 (November): 1181–99.