| CHEMICAL ENGINEERING TRANSACTIONS VOL. xxx, 2026 | A publication of |
| --- | --- |
|  | The Italian Association of Chemical Engineering Online at www.cetjournal.it |
| ### Guest Editors: Copyright © 2026, AIDIC Servizi S.r.l. ISBN 979-12-81206-XX-X; ISSN 2283-9216 |  |

Kinetic modeling of ABE fermentation in mono- and co-cultures of Clostridium saccharobutylicum DSM 13864 and Clostridium pasteurianum DSM 525

Bruno X. Ferreira, Pedro Vasconcellos Cazzoli, Riann Q. Nóbrega, Rubens Maciel Filho, Carla F. S. Vieira, Adriano P. Mariano*

Universidade Estadual de Campinas (UNICAMP), Faculdade de Engenharia Química (FEQ), Laboratory of Optimization, Design, and Advanced Control – Fermentation Division (LOPCA-Ferm), Campinas, SP, Brazil.

*e-mail: adpm@unicamp.br

Acetone–butanol–ethanol (ABE) fermentation is the primary method for producing butanol from renewable feedstocks. Butanol is used in several applications, notably as a precursor for acrylates and as an automotive fuel. Clostridium species perform ABE fermentation using a variety of carbon sources. A kinetic model that predicts butanol production dynamics is valuable for assessing industrial feasibility and for gaining insights that can guide future process improvements. This study adjusts kinetic models of butanol production using two carbon sources (glucose or xylose) and two Clostridium species (C. saccharobutylicum DSM 13864 and C. pasteurianum DSM 525). Six fermentations were performed (in triplicate) in synthetic P2 medium at 35°C with an initial sugar concentration of 60 g/L. For each sugar, three conditions were tested: monocultures of each Clostridium strain and a coculture. The predictive models (mass balances and reaction rates) were taken from the literature. Parameter fitting was performed using the library SciPy (function scipy.optimize.least_squares). The coculture results were not statistically different from the C. saccharobutylicum monoculture (Tukey test, p = 0.05), suggesting that C. saccharobutylicum dominates the fermentation and does not show a significant improvement with the coculture under the tested conditions. The adjusted kinetic models presented the best results in modeling the sugar consumption values (R2 > 80 %); however, further modification of the equation will be investigated to improve the fit on the butanol concentration.

Introduction

Biobased products are becoming more present and valorized with the global awareness about the sustainability and environmental impact of the compounds used in different segments, such as bioplastics, biomaterials, and biofuels (Gupta et al., 2022). In that global context, the butanol produced from renewable resources, also called biobutanol, stands out as a prominent substance due to its multiple uses, most commonly applied as solvents, coatings, but also could be used as biofuels, as an alternative to bioethanol, and as an ingredient in the pharmaceutical and cosmetics industries. These diverse uses have driven a projected growth of 5.44 % CAGR (Compound Annual Growth Rate) until 2035 (Market Growth Reports, 2026; Coherent Market Insights, 2025).

Biobutanol is usually obtained through an anaerobic fermentation called ABE (Acetone-Butanol-Ethanol), in which the principal product is the C4 alcohol. In most cases, this process is conducted with Clostridium bacteria species, which are able to use different kinds of carbon sources (such as glucose, xylose, and glycerol) to grow and generate butanol (Zetty-Arenas et al., 2019). A strategy explored to improve the butanol production in ABE fermentations is co-culturing, which can be with microorganisms of different species or between two strains of Clostridium (Pinto et al., 2021).

An approach to improve the understanding of the behavior of the bacteria during the coculture is to adjust mathematical models, whether phenomenological (Rodrigues et al., 2016) or constructed from data (Kumar et al., 2024). This work aims to investigate the possible gains from the coculture of two Clostridium species, C. saccharobutylicum and C. pasteurianum, and adjust the parameters of kinetic models for mono- and coculture fermentations.

Methodology

Fermentation

Clostridium saccharobutylicum DSM 13864 and C. pasteurianum DSM 525 were obtained from the German Collection of Microorganisms and Cell Cultures (DSMZ) in Braunschweig, Germany. Their inoculum was prepared from a 400-μL aliquot of the spores’ suspension, stored at -4 ºC, and added to a 10-mL anoxic pre-sterilized TGY medium (30 tryptone, 20 glucose, and 10 yeast extract [g·L-1]). The pre-inoculum mixture was incubated in an anaerobic chamber (COY Type A vinyl chamber) at 35 °C under an atmosphere of 98% nitrogen and 2% hydrogen, for 16 h (C. saccharobutylicum DSM 13864) and 20 h (C. pasteurianum DSM 525). Then, the pre-inoculum was transferred to 90 mL of TGY medium under the same conditions for 6 h (C. saccharobutylicum DSM 13864) and 12 h (C. pasteurianum DSM 525).

Fermentation experiments (Table 1) were conducted in 250-mL loosely capped bottles (in triplicate), with a total working volume of 100 mL per bottle, using the P2 medium (Vieira et al., 2020) with 60 g·L-1 of sugar (glucose or xylose) as initial concentration (sterilized using autoclave (121 ºC; 15 min) or filtered with a 0.22 μm filter). The experiments were conducted under the same conditions as the inoculum, with 10 % (v/v) inoculum in the monoculture or 5 % (v/v) inoculum of each species in the co-culture. Samples were collected at 0 h, 2 h, 4 h, 6 h, 9 h, and 12 h, then every 12 h up to 96 h for analysis.

Table 1: Codes for the various conditions of fermentation (a - C. saccharobutylicum DSM 13864; b - C. pasteurianum DSM 525; c – co-culture)

| Carbon source (initial concentration [g·L-1]) | ID |
| --- | --- |
| Glucose (60) | 01a, 03b, 05c |
| Xylose (60) | 02a, 04b, 06c |

Modeling

For the modeling, the reaction rate and mass balance equations were obtained based on the equations presented by Rochón et al. (2017) and Plaza et al. (2022), using ODEs to predict the cell growth (EQ.(3)), substrate consumption (EQ (2)), and the main products generation (EQ (4-6)). Parameters were adjusted using the library SciPy (function scipy.optimize.least_squares, v1.16.2) in the Google Colaboratory environment (with Python v.3.10). Two approaches were tested, both using the same equations for the cell growth rate (Eq (1)), with a butanol inhibition and death cells parameter modifying the Monod Kinetic equations, and the substrate consumption. Model 1 (M1) only predicted the butanol formation (Eq (4)), while Model 2 (M2) added the mass balance equations for the acetone (EQ (5)) and ethanol (EQ (6)) production.

| μ= μmax∙Sks+S∙1-BkIBα-kd | (1) |
| --- | --- |
| dSdt= -μ∙X∙1YX/S | (2) |
| dXdt= μ∙X | (3) |
| dBdt= μ∙X∙YB/SYX/S | (4) |
| dAdt= μ∙X∙YA/SYX/S | (5) |
| dEdt= μ∙X∙YE/SYX/S | (6) |

where X is concentration of cell (g·L-1), µ is the specific growth rate (h-1), µmax is the maximum specific growth rate (h-1), S is the substrate concentration (g·L-1), ks is the substrate saturation constant (g·L-1), kd is the specific cell death rate (h-1), B is the butanol concentration (g·L-1), kIB is the butanol concentration that limited the cell growth (g·L-1), α is the degree of product inhibition, YX/S is the biomass yield coefficient (g·g-1), YB/S is the butanol yield coefficient (g·g-1), A is the acetone concentration (g·L-1), YA/S is the acetone yield coefficient (g·g-1), E is the ethanol concentration (g·L-1), and YE/S is the ethanol yield coefficient (g·g-1).

Analysis

The concentrations of substrates and products were measured throughout the fermentation using High-Performance Liquid Chromatography (HPLC - Agilent 1260 Infinity) (Chacón et al. 2020). Cell concentration was monitored by optical density (600 nm) using a spectrophotometer (LGI-VS0721N) and converted to concentration in g/L and mol/L using the correlations presented by Raganati et al. (2015) and Shinto et al. (2007). To evaluate the fermentation, the fermentation productivity parameters (g·(L∙h)-1), product yield (g·g-1), and sugar conversion (%) were calculated, and the difference between the mean values ​​of the treatments was compared using the Tukey test (p < 0.05). Regarding the accuracy of the model and the parameters adjusted for the co-culture, they were evaluated using the root mean squared error (RMSE) and the coefficient of determination (R²).

Results and Discussions

Fermentation

Analyzing the cells’ growth (Figure 1), all the fermentations had their exponential phase after 4 h, except for experiment 05, which also had the lowest maximum cell concentration (Table 2). Also, comparing monocultures with C. pasteurianum (Figure 1 C-D), the one with glucose consumption was faster; however, the xylose utilization was over 70 % higher. Although both sugar consumptions were lower than 40 %, this is reflected in the ABE production (Table 2). The behavior of the co-culture experiments (Figure 1 E-F) was similar to that of monoculture with C. saccharobutylicum (Figure 1 A-B), indicating that this species was predominant (further investigation will be necessary to confirm).

Figure 1: Sugars, products, and cell concentration through the fermentations (OD – Optical Density).

As expected, the butanol concentration plays an important role in carbon source consumption and cell growth. Comparing the butanol yield and productivity (Table 2), no difference was observed comparing the C. saccharobutylicum and the co-culture experiments, indicating that the coculture under the test conditions did not provide a significant gain to the process. Meanwhile, other conditions for this co-culture could be tested (such as varying inoculation times, mixing the carbon sources, raising the temperature to 37 ºC, and supplementing the medium with a calcium source to favor C. pasteurianum), aiming to explore the maximum potential of this co-culture.

Table 2: Main parameters calculated from the data collected from the fermentations (fermentation time 96 h, except for Xmax)

|  | Experiments |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 02 | 03 | 04 | 05 | 06 |
| ABE final [g·L-1] | 12.96a | 12.76a | 4.54b | 3.33b | 11.05a | 9.95a |
| Butanol final [g·L-1] | 7.49a | 7.71a | 0.24 | 1.28 | 7.40a | 7.03a |
| Ethanol final [g·L-1] | 3.11a | 3.06a | 2.83a | 2.03a | 1.99a | 1.66a |
| Acetone final g·L-1] | 2.64a | 2.30a,b,c | 1.37b.c | 0.02 | 1.80b,c | 1.32c |
| Sugar utilized [g·L-1] | 35.81a | 37.63a | 12.44 | 22.37 | 40.38a | 35.99a |
| Residual sugar [g·L-1] | 20.07a | 20.44a | 43.85 | 36.15 | 16.43 | 20.94a |
| % substrate utilized | 64.08a | 64.80a | 22.10 | 38.23 | 71.08a | 63.21a |
| Maximum cells (Xmax) [g·L-1] | 2.81a | 2.74a | 2.26 | 1.62 | 2.75a | 2.59a |
| ABE yield (YABE/S) [g·g-1] | 0.36a | 0.34a | 0.36a | 0.15a | 0.27a | 0.28a |
| Butanol yield (YB/S) [g·g-1] | 0.21a | 0.20a | 0.02b | 0.06b | 0.18a | 0.20a |
| Ethanol yield (YE/S) [g·g-1] | 0.09a.b | 0.08a | 0.23b | 0.09a.b | 0.05a | 0.05a |
| Acetone yield (YA/S) [g·g-1] | 0.07a | 0.06a | 0.11 | 0.00a | 0.04a | 0.04a |
| ABE productivity [g·(L∙h)-1] | 0.13a | 0.13a | 0.05b | 0.03b | 0.12a | 0.10a |
| Butanol productivity [g·(L∙h)-1] | 0.078a | 0.080a | 0.002 | 0.013 | 0.077a | 0.073a |
| a, b, c – Means followed by different letters are significantly different from each other - Tukey test (p-value < 0,05) |  |  |  |  |  |  |

Modeling

For both models, the highest fitting results (Table 4) were obtained for the glucose values (Table 3), indicating that the butanol inhibition terms chosen for EQ (1) were adequate for the fermentation conditions. For experiment 03, the models adjusted for X and butanol concentration prediction had the worst results, with R2 < 55 %. It is probably associated with the low butanol production, making the model inadequate for the experiment, and indicating that, for this monoculture, another set of mass balance ODEs must be explored in future works. For the other experiments, the butanol concentration prediction has an R2 value above 70 %, indicating that the models are able to predict product formation behavior; however, they still need improvement for practical application. Another interesting point is that including the ODE equations to predict acetone and ethanol generation did not lead to significant losses in fitting performance (Table 3). Although in some cases the parameters changed significantly (Table 4), this could indicate overfitting and should be evaluated in future work using more experimental data. The variations, in addition to R2 values below 80 %, indicate that the models need to be improved before being applied in other studies, such as technical-economic analysis, to provide more reliable information about the microorganism’s behavior.

Table 3: Performance results for the models M1 and M2 for the fermentation with the adjusted kinetic parameters.

|  |  |  | Experiments |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Criteria | Compound | Model | 01 | 02 | 03 | 04 | 05 | 06 |
| R2 | Sugar | M1 | 0.984 | 0.981 | 0.911 | 0.968 | 0.847 | 0.250 |
|  |  | M2 | 0.977 | 0.920 | 0.962 | 0.965 | 0.958 | 0.955 |
|  | X | M1 | 0.797 | 0.780 | 0.280 | 0.867 | 0.880 | 0.652 |
|  |  | M2 | 0.838 | 0.459 | 0.215 | 0.962 | 0.696 | 0.373 |
|  | Butanol | M1 | 0.811 | 0.92 | 0.476 | 0.883 | 0.752 | 0.711 |
|  |  | M2 | 0.793 | 0.82 | 0.534 | 0.843 | 0.899 | 0.877 |
|  | Acetone | M1 | - | - | - | - | - | - |
|  |  | M2 | 0.793 | 0.82 | 0.969 | 0.229 | 0.220 | 0.877 |
|  | Ethanol | M1 | - | - | - | - | - | - |
|  |  | M2 | 0.572 | 0.525 | 0.677 | 0.342 | 0.497 | 0.444 |

Table 4: Kinetic parameters adjusted for each experiment using both models.

|  |  | Kinect parameters |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Model | µmax | kS | kIB | α | kd | YX/S | YB/S | YA/S | YE/S |
| 01 | M1 | 0.50 | 14.87 | 20.00 | 0.01 | 0.35 | 0.08 | 0.04 | - | - |
|  | M2 | 1.29 | 119.9 | 6.43 | 1.5 | 0.0001 | 0.71 | 0.173 | 0.398 | 0.029 |
| 02 | M1 | 0.60 | 6.00 | 7.50 | 1.70 | 1.00E-05 | 0.06 | 0.20 | - | - |
|  | M2 | 1.27 | 119.91 | 6.36 | 1.49 | 0.0001 | 0.07 | 0.201 | 0.056 | 0.079 |
| 03 | M1 | 0.79 | 11.62 | 0.50 | 0.62 | 0.50 | 0.14 | 0.01 | - | - |
|  | M2 | 0.42 | 9.06 | 0.196 | 0.12 | 0.369 | 0.1164 | 0.014 | 0.112 | 0.183 |
| 04 | M1 | 0.50 | 14.87 | 20.00 | 0.01 | 3.52 | 0.08 | 0.04 | - | - |
|  | M2 | 0.63 | 8.0 | 25.0 | 0.10 | 0.510 | 0.7 | 0.05 | 0.02 | 0.08 |
| 05 | M1 | 0.70 | 19.93 | 7.50 | 1.90 | 3.44E-04 | 0.07 | 0.23 | - | - |
|  | M2 | 1.06 | 119.98 | 7.85 | 1.5 | 0.0001 | 0.07 | 0.20 | 0.047 | 0.041 |
| 06 | M1 | 0.69 | 5.00 | 5.50 | 0.24 | 0.30 | 0.06 | 0.18 | - | - |
|  | M2 | 1.02 | 119.57 | 5.98 | 1.4991 | 0.0001 | 0.51 | 0.18 | 0.036 | 0.031 |

Conclusions

This study evaluated cocultures of two Clostridium species to determine whether coculturing increases butanol production compared with monocultures. When either glucose or xylose was used as the carbon source, cocultures did not produce significantly more butanol and behaved similarly to the C. saccharobutylicum monoculture. Fermentation conditions likely favored dominance by C. saccharobutylicum rather than a synergistic interaction between the species. Kinetic modeling was performed for mono- and cocultures using two models. The second model only predicted acetone and ethanol concentrations from the initial values and did not substantially reduce prediction errors for substrates, biomass, or butanol. Overall, sugar consumption was predicted best (R2 > 84%), while experiment 03 (glucose – monoculture - C. pasteurianum) showed the poorest fit for biomass and butanol (R2 < 50%), likely because its low butanol production made the model unsuitable. Future work will test modified fermentation conditions (e.g., temperature and inoculation timing) to determine whether coculture performance can be improved, and will explore alternative model equations to enhance predictive performance on both current and new coculture data.

Data availability

The codes used on the adjustment of the models’ parameters and their values are available at the GitHub page (https://github.com/FerreiraBX95/Ferreira-et-al.---IConBM_2026.git)

Acknowledgments

This study was financed, in part, by the São Paulo Research Foundation (FAPESP), Brazil (process numbers 2023/01072-7, 2023/01219-8, and 2024/07704-8). We also thank the financial support from Coordenação de Aperfeiçoamento de Pessoal de Nível Superior – Brasil (CAPES/PROEX).

References

Chacón S. J., Matias G., Vieira C. F. dos S., Ezeji T. C., Maciel Filho R., Mariano, A. P., 2020, Enabling butanol production from crude sugarcane bagasse hemicellulose hydrolysate by batch-feeding it into molasses fermentation, Industrial Crops and Products, v. 155.

Coherent Market Insight, 2025, Bio-butanol market analysis & forecast: 2025-2032 <https://www.coherentmarketinsights.com/market-insight/bio-butanol-market-696> accessed 20.01.2026.

Gupta V. K., Pandey A., Koffas M. Mussato A. I., Khare S., 2022, Biobased biorefineries: Sustainable bioprocesses and bioproducts from biomass/bioresources special issue, Renewable and Sustainable Energy Reviews, v. 167.

Kumar K., Jadhav S. M., Moholkar S., Acetone-Butanol-Ethanol (ABE) fermentation with clostridial co-cultures for enhanced biobutanol production. Process Safety and Environmental Protection, v. 185, p. 277-285, 2024.

Market Growth Reports, 2026, Biobutanol Market Size, Share, Growth, and Industry Analysis, By Type (Biobutanol Liquids, Biobutanol Powder), By Application (Biofuels, Industrial Solvents, Pharmaceuticals, Cosmetic Products), Regional Insights and Forecast to 2035, <https://www.marketgrowthreports.com/market-reports/biobutanol-market-113919> accessed 20.01.2026.

Pinto T., Flores-Alsina X., Gernaey K. V., Junicke H., 2021, Alone or together? A review on pure and mixed microbial cultures for butanol production, Renewable and Sustainable Energy Reviews, v. 147.

Plaza, P. E., Coca M., Yagüe S. L., Gutiérrez G., Rochón E., García-Cubero, M. T., 2022, Bioprocess intensification for acetone-butanol-ethanol fermentation from brewer’s spent grain: Fed-batch strategies coupled with in-situ gas stripping, Biomass and Bioenergy, v.156.

Raganati F., Procentese A., Olivieri G., Götz P., Salatino P., Salatin A., 2015, Kinetic study of butanol production from various sugars by Clostridium acetobutylicum using a dynamic model, Biochemical Engineering Journal, v. 99, p. 156-166.

Rochón E., Ferrari M. D., Lareo C., 2017, Integrated ABE fermentation-gas stripping process for enhanced butanol production from sugarcane-sweet sorghum juices, Biomass and Bioenergy, v. 98, p. 153-160.

Rodrigues B., Lima-Costa M. E., Constantino A., Raposo S., Felizardo C., Gonçalves D., Fernandes T., Dionísio L., Peinado J. M., 2016, Growth kinetics and physiological behaviour of co-cultures of Saccharomyces cerevisiae and Kluyveromyces lactis, fermenting carob sugars extracted with whey, Enzyme and Microbial Technology, v. 92, p. 41-48.

Shinto H., Tashiro H., Yamashita M., Kobayashi G., Sekiguchi T., Hanai T., 2007, Kinetic modeling and sensitivity analysis of acetone–butanol–ethanol production, Journal of Biotechnology, v. 131, p. 45–56.

Vieira C.F.S., Maugeri Filho F., Maciel Filho R., Pinto Mariano A., 2020, Isopropanol-butanol-ethanol (IBE) production in repeated-batch cultivation of Clostridium beijerinckii DSM 6423 immobilized on sugarcane bagasse, Fuel, v. 263.

Zetty-Arenas A. M., Alves R. F., Portela C. A. F., Mariano A. P., Basso T. O., Tovar L. P., Filho R. M., Freitas S, 2019, Towards enhanced n-butanol production from sugarcane bagasse hemicellulosic hydrolysate: Strain screening, and the effects of sugar concentration and butanol tolerance, Biomass and Bioenergy, v. 126, p. 190–198.