| CHEMICAL ENGINEERING TRANSACTIONS VOL. , 2025 | A publication of |
| --- | --- |
|  | The Italian Association of Chemical Engineering Online at www.cetjournal.it |
| ### Guest Editors: Copyright © 2025, AIDIC Servizi S.r.l. ISBN 979-12-81206-XX-X; ISSN 2283-9216 |  |

Global Synthesis Model for Hydrogen Production Systems and High Value-added Compounds from Glycerol

Tayla Luiza P. Borges*, Reginaldo Guirardello

School of Chemical Engineering – Unicamp, Av. Albert Einstein 500, Campinas–SP, 13083-852, Brazil

t223350@dac.unicamp.br

The energy transition has driven the development of sustainable routes for hydrogen production, due to the dependence on fossil fuel-based methods and their high emissions. Glycerol, an abundant and low-cost by-product of the biodiesel industry, stands out as a promising feedstock for hydrogen generation and higher value-added products. This work proposes a global synthesis methodology, formulated as a linear programming model, to identify high-value products and hydrogen as a co-product, using purified glycerol and ensuring zero CO₂ emissions. Eight scenarios were evaluated with the objective of maximizing the sale price, varying the fixed hydrogen production (nH₂) and, in specific scenarios (4-8), applying additional restrictions on the use of raw materials. All scenarios met the imposed restrictions. Scenario 2 exhibited the best economic performance, with a selling price of US$ 30,960, an enthalpy of –18,600 kJ, and the production of glycolic acid (150 kmol) and hydrogen (100 kmol). Scenario 7 showed greater product diversity, including the formation of acrylonitrile (10 kmol) and higher hydrogen production (200 kmol), resulting in a financial return of US$ 21,240. Thus, the model demonstrated the ability to identify diverse and viable combinations for converting glycerol into industrially relevant compounds and hydrogen.

Introduction

In recent years, the need to reduce emissions and mitigate the impacts of climate change has driven the search for cleaner energy matrices. Among these, hydrogen has emerged as a strategic vector for decarbonization due to its high energy density (Ishaq et al., 2022). Although abundant in nature, less than 1% is found in it molecular form (H₂), and its production is still predominantly based on fossil routes, such as methane steam reforming, which leads to greenhouse gas (GHG) emissions (Gujar et al., 2025). Green alternatives, such as water electrolysis, remain limited by their high cost and substantial energy demand (Odenweller and Ueckerdt, 2025).

In this context, glycerol has gained attention as a promising feedstock for hydrogen production. As a byproduct of biodiesel production, it accounts for approximately 10 wt% of the total output (Khor et al., 2022). In its purified form, glycerol has applications in the food, cosmetic, and pharmaceutical sectors. However, the rapid expansion of the biodiesel sector has increased glycerol supply far beyond industrial demand, reducing its commercial value. Thus, its conversion into higher value-added products and hydrogen generation has emerged as a promising strategy for valorization (Sandid et al., 2024).

Glycerol conversion into hydrogen can occur through routes such as steam reforming, autothermal reforming, and gasification. Nevertheless, these technologies still require high energy inputs and generate significant emissions, which limit their efficiency (Rastegari et al., 2018) and reinforce the need for more sustainable and efficient alternatives.

The objective of this work is to develop a global process synthesis methodology capable of identifying products with high profit margins, using purified glycerol as feedstock and hydrogen as a co-product, under zero CO₂ production in the global reaction. The model is formulated as a linear programming problem, based on atomic balance equations, non-negativity constraints, and a global enthalpy requirement that ensures an exothermic process.

Methodology

The mathematical formulation follows the linear programming model presented by Silva et al. (2025), in which only glycerol (C₃H₈O₃) is pre-defined as the feedstock and hydrogen as the desired co-product. The other reactants and products are not initially specified, being determined by the model itself during optimization. The objective function aims to maximize the final selling price of the process, as expressed in Eq. (1). The calculation of the final selling price is given by Eq. (2), where, ni is defined as the number of moles of substance i (kgmol), pi as the individual price of substance i (US$/kg) e mmi as its molar mass (kg/kgmol).

| maxni Selling Price | (1) |
| --- | --- |

| Selling Price= (pi . ni . mmi) | (2) |
| --- | --- |

The system incorporates the atomic balance constraint (Eq. 4) and the global enthalpy constraint (Eq. 5), ensuring thermodynamic consistency and that the global process is non-endothermic. Similarly, the non-negativity constraint (Eq. 3) is applied, preventing the model from generating negative molar values and preserving the physical meaning of the process.

| ni≥0 | (3) |
| --- | --- |

| i=1Nami∙ niproducts=i=1Nami∙nireagents | (4) |
| --- | --- |

| ∆HRglobalT0,P0=i=1Nniproducts∙hi(T0,P0)-i=1Nnireagents∙hi(T0,P0)≤0 | (5) |
| --- | --- |

The term ami represents the number of atoms of element m in substance i; ∆HRglobal is the global reaction enthalpy; T0,P0 correspond to standard temperature and pressure conditions (25 °C and 1 atm); and hi corresponds to the specific enthalpy of substance i. The enthalpy of formation values were obtained from thermodynamic tables available in the literature (CRC Handbook, 2003).

Case Study

The substances considered in the model were divided into potential reactants and products. The feedstocks evaluated were:

Glycerol (C3H8O3);

Oxygen (O2);

Nitrogen (N2);

Water (H2O).

The potential products were selected based on the literature, encompassing species obtained from glycerol through known thermochemical routes such as oxidation, hydrogenolysis, and dehydration (Koronian et al., 2022). In total, fifteen products were considered, including hydrogen. Table 1 shows the chemical formula, standard enthalpy of formation, and unit prices, along with the reference year adopted, for each substance considered in the model.

Table 1: Chemical formula and standard enthalpy of formation of the substances and unit price

| Component | Formula | H°f (kJ/mol) | Price (US$/kg) | Reference year | Source |
| --- | --- | --- | --- | --- | --- |
| Formic acid | CH2O2 | -425.00 | 0.44 | 2024 | Sunsirs (2025) |
| Glycolic acid | C2H4O3 | -583.00 | 2.68 | 2022 | PMarket research (2025) |
| Lactic acid | C₃H₆O₃ | -674.50 | 1.15 | 2024 | Imarc (2025a) |
| Methanol | CH₃OH | -239.20 | 0.22 | 2023 | Methanex (2025) |
| 1,2-propanediol | C3H8O2 | -421.50 | 1.52 | 2023 | Imarc (2025b) |
| Acetone | C₃H₆O | -248.40 | 1.22 | 2024 | Imarc (2025c) |
| 1-propanol | C3H8O | -302.60 | 1.3 | 2024 | Echemi (2025) |
| 2-propanol | C3H8O | -272.10 | 1.28 | 2024 | Imarc (2025d) |
| Propylene | C3H6 | 20.23 | 0.87 | 2022 | Statista (2025a) |
| Propane | C3H8 | -103.80 | 0.46 | 2025 | EIA (2025) |
| Acrolein | C3H4O | -85.80 | 1.4 | 2023 | Vargas, B.J et al. (2023) |
| Acrylonitrile | C3H3N | 185.00 | 1.3 | 2024 | Statista (2025b) |
| Acrylic acid | C3H4O2 | -355.90 | 1.40 | 2024 | Imarc (2025e) |
| Ethylene glycol | C₂H₆O₂ | -392.20 | 0.72 | 2022 | Sunsirs (2022) |
| Hydrogen | H2 | 0.00 | 1.70 | 2023 | Curcio (2025) |

Scenario Construction

Eight optimization scenarios were evaluated using the same objective function and database, with a fixed glycerol feed of 100 kgmol (nC₃H₈O₃ = 100).

In Scenarios 1–3, only the fixed hydrogen production (nH₂) was varied. To evaluate the influence of the water-oxygen ratio, Scenarios 4–6 were formulated by imposing simultaneous restrictions on the use of O₂ and H₂O. Finally, since N₂ was not selected in the initial scenarios, Scenarios 7 and 8 included a minimum usage restriction for this component, aiming to evaluate its effect and the possible formation of new compounds.

Scenario 1: nH2=0;

- Scenario 2: nH2=100;

Scenario 3: nH2=200;

Scenario 4: nO₂ ≤ 5; nH₂O ≥ 30; nH2=100;

Scenario 5: nO₂ ≤ 10; nH₂O ≥ 30; nH2=100;

Scenario 6: nO₂ ≤ 15; nH₂O ≥ 30; nH2=100;

Scenario 7: nN₂ ≥ 5; nH₂O ≥ 50; nH2=200;

Scenario 8: nN₂ ≥ 10; nH₂O ≥ 30; nH2=200.

Result and Discussion

Table 2 summarizes the enthalpy variation and selling price obtained for each optimization scenario. The detailed molar composition of reactants and products is presented in Table 3.

Table 2: Global enthalpy and final selling price

| Scenario | Enthalpy ∆H (kJ) | Selling price |
| --- | --- | --- |
| 1 | -13,370 | 21,146 |
| 2 | -18,600 | 30,960 |
| 3 | 0.00 | 27,882 |
| 4 | 0.00 | 8,968 |
| 5 | 0.00 | 13,297 |
| 6 | 0.00 | 17,627 |
| 7 | 0.00 | 21,240 |
| 8 | 0.00 | 14,599 |

Table 3: Results of the scenarios

| Scenario | Reactants |  |  |  |  | Products |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | C₃H₈O₃ | O2 |  |  |  | C2H4O3 | CH3OH |  |  |  |
|  | n=100 | n=50 |  |  |  | n=100 | n=100 |  |  |  |
| 2 | C₃H₈O₃ | O2 |  |  |  | C2H4O3 | H2 |  |  |  |
|  | n=100 | n=75 |  |  |  | n=150 | n=100 |  |  |  |
| 3 | C₃H₈O₃ | O2 | H2O |  |  | CH2O2 | C2H4O3 | H2 |  |  |
|  | n=100 | n=35,5 | n=100 |  |  | n=41.82 | n=129.10 | n=200 |  |  |
| 4 | C₃H₈O₃ | O2 | H2O |  |  | CH2O2 | C2H4O3 | C3H6O3 | C3H8O | H2 |
|  | n=100 | n=5 | n=30 |  |  | n=96.94 | n=3.06 | n=35.64 | n=30 | n=100 |
| 5 | C₃H₈O₃ | O2 | H2O |  |  | CH2O2 | C2H4O3 | C3H6O3 | C3H8O | H2 |
|  | n=100 | n=10 | n=30 |  |  | n=76.98 | n=33.01 | n=22.33 | n=30 | n=100 |
| 6 | C₃H₈O₃ | O2 | H2O |  |  | CH2O2 | C2H4O3 | C3H6O3 | C3H8O | H2 |
|  | n=100 | n=15 | n=30 |  |  | n=57.04 | n=62.96 | n=9,01 | n=30 | n=100 |
| 7 | C₃H₈O₃ | O2 | H2O | N2 |  | CH2O2 | C2H4O3 | C3H3N | H2 |  |
|  | n=100 | n=33.5 | n=85 | n=5 |  | n=94.01 | n=87.99 | n=10 | n=200 |  |
| 8 | C₃H₈O₃ | O2 | H2O | N2 |  | CH2O2 | C2H4O3 | C3H3N | H2 |  |
|  | n=100 | n=31.5 | n=70 | n=10 |  | n=146.20 | n=46.89 | n=20 | n=200 |  |

n= number of moles (kgmol)

In Scenario 1, the model resulted in the formation of glycolic acid (C₂H₄O₃) and methanol (CH₃OH), configuring a strongly exothermic process. Glycolic acid was selected due to its high unit value and compatibility with the atomic balance, being one of the main C₂ products derived from glycerol oxidation (Koronian et al., 2022). Tavera Ruiz et al. (2021) reported its production with 100% glycerol conversion using cerium–zirconium catalysts.

Methanol, although having a lower individual value, contributes to closing the H, C, and O balances and appears in larger amounts due to its low molar mass, increasing the total value of the scenario. Industrially, it can be produced from syngas generated by the reforming or partial oxidation of glycerol (Lin, 2013).

In Scenario 2, the requirement to generate 100 kmol of H₂ alters the global balance and directs the solution primarily toward the formation of glycolic acid. This compound has a high unit value and is obtained in a greater amount than in Scenario 1, which, combined with the additional hydrogen generation, significantly increases the final process value.

Furthermore, this scenario presents the most exothermic reaction among those evaluated (-18,600 kJ), indicating good energy viability. As a result, the combination of high economic value per unit and stoichiometric viability makes glycolic acid the optimal solution for maximizing the final price when moderate hydrogen production is required.

In Scenario 3, the requirement of a fixed production of 200 kmol of hydrogen changes the optimal configuration of the system. Since the hydrogen contained in glycerol is not sufficient to meet this demand, the model incorporates water as a reactant to supply additional hydrogen, adjusting the H and O balances without any relevant economic impact.

In this more oxidizing scenario, in addition to glycolic acid, formic acid (CH₂O₃) is also formed, a compound compatible with the lower H/C ratio required. Industrially, this compound is mainly obtained through the selective oxidation of glycerol. Xu et al. (2014) reported that metallic catalysts such as Ru(OH)₄/r-GO and FeCl₃ can direct glycerol oxidation toward formic acid, achieving conversions of up to 95% and yields above 50%.

Scenarios 4–6, despite the different proportions of oxygen and water, resulted in the same set of products, varying only in the molar quantities obtained. This behaviour reflects the role of oxygen in the redistribution of oxygenated products according to their availability, while water primarily acts to adjust the hydrogen balance of the system. The additional formation of 1-propanol (C₃H₈O) was observed, remaining almost constant and accommodating the excess hydrogen introduced by the minimum water restriction.

Another compound present is lactic acid (C₃H₆O₃), which acts as an economically viable alternative for maximizing the objective function when the formation of glycolic acid (C₂H₄O₃), one of the products with the highest unit value considered, is limited by oxygen availability. The feasibility of converting glycerol into lactic acid has been widely reported in the literature. Venkateshappa et al. (2024) demonstrated high yields and selectivity in the production of lactic acid from glycerol using iron catalytic complexes, highlighting the potential of this compound as a high value-added product derived from glycerol.

As oxygen availability increases, as observed in Scenario 6, the system progressively approaches the behaviour observed in Scenario 3, resulting in higher final selling prices. This trend is associated with the enhanced formation of glycolic acid (C₂H₄O₃), thereby improving the overall economic attractiveness of the system.

Overall, Scenarios 4-6 can be interpreted as restricted adaptations of Scenario 3, preserving the trend toward economic optimization while adjusting the distribution of products under feedstock availability constraints imposed on the system.

Scenarios 7 and 8 introduce a new perspective to the system by imposing a minimum restriction on N₂ use, which leads to the formation of a new compound, acrylonitrile (C₃H₃N), without compromising high H₂ production. The production of acrylonitrile from glycerol has attracted growing industrial interest. Guerra et al. (2023) proposed obtaining renewable acrylonitrile from glycerol using green propylene as an intermediate, highlighting the versatility of this route for converting renewable feedstocks into high value-added nitrogenous products.

In Scenario 7, N₂ acts as a secondary component, resulting in limited acrylonitrile formation. In contrast, in Scenario 8, the increased availability of N₂ intensifies the formation of C₃H₃N, demonstrating the system's ability to direct carbon to nitrogenous products as nitrogen availability increases.

Despite the higher production of acrylonitrile in Scenario 8, the final price of the process is lower than that observed in Scenario 7. This behaviour is associated with a reduction in the formation of glycolic acid (C₂H₄O₃) and an increase in the production of formic acid (CH₂O₂), which has a lower economic value. The inclusion of N₂ in the system enables the formation of high value-added nitrogen compounds, expanding the possibilities for glycerol valorization.

Although these results highlight the potential of the proposed pathways, a detailed estimation of investment costs was not considered at this stage and will be evaluated in future work, including process simulation in Aspen Plus and a techno-economic analysis of the scenarios.

Conclusions

This work presented a global synthesis methodology to identify economically attractive reaction routes using purified glycerol as feedstock and hydrogen as a coproduct under zero CO₂ production conditions. The linear programming model enabled the determination of optimal combinations of reactants and products.

All evaluated scenarios satisfied atomic balance, non-negativity, and global enthalpy constraints, resulting in exothermic processes. Scenario 2 exhibited the best economic performance, with a fixed hydrogen production of 100 kmol, a financial return of US$ 30,960, and the highest energetic feasibility (–18,600 kJ).

Although Scenario 7 presents a lower final selling price than Scenario 2, it stands out due to its greater product diversity and higher hydrogen production (200 kmol). Additionally, the formation of acrylonitrile, even in low amounts, highlights the potential of the system to produce nitrogen-containing compounds of industrial interest, thereby expanding the possibilities for glycerol valorization. The formation of glycolic acid, observed in all scenarios, was decisive for economic viability due to its high unit value and significant production. Overall, the model proved to be an efficient tool for selecting viable reactants and products, contributing to glycerol valorization and highlighting the potential of hydrogen as a coproduct in maximizing the overall process value.

Acknowledgments

This study was supported by Coordenação de Aperfeiçoamento de Pessoal de Nível Superior - CAPES, Brazil - Finance Code 001 (Grant n. 88887.132544/2025-00), and by the São Paulo Research Foundation - FAPESP (Grant n. 2022/16379-8).

References

Curcio E., 2025, Techno-economic analysis of hydrogen production: Costs, policies, and scalability in the transition to net-zero. International Journal of Hydrogen Energy, 128, 473–487.

Echemi, 2025, 1-Propanol Price Database <echemi.com/price-database/1-propanol-pd20170111160501595.html> accessed 05.12.2025.

Guerra G.N., Neto G.J.M., Alves R.M.B., Pontes L.A.M., 2023, Conceptual process and surrogate optimization of acrylonitrile production from glycerol via green propylene, Brazilian Journal of Chemical Engineering, 41, 901–910.

Gujar J. P., Verma A., Modhera B., 2025, Optimizing glycerol conversion to hydrogen: A critical review of catalytic reforming processes and catalyst design strategies, International Journal of Hydrogen Energy, 109, 823–850.

IMARC Group, 2025a, Lactic Acid Pricing Report: Market Trends, Share, Size, Growth, Opportunity and Forecast 2025–2030 <imarcgroup.com/lactic-acid-pricing-report> accessed 04.12.2025.

IMARC Group, 2025b, Propylene Glycol Pricing Report: Market Trends, Share, Size, Growth, Opportunity and Forecast 2025–2030 <imarcgroup.com/propylene-glycol-pricing-report> accessed 05.12.2025.

IMARC Group, 2025c, Acetone Pricing Report: Market Trends, Share, Size, Growth, Opportunity and Forecast 2025–2030. <imarcgroup.com/acetone-pricing-report> accessed 05.12.2025.

IMARC Group, 2025d, Isopropyl Alcohol Pricing Report: Market Trends, Share, Size, Growth, Opportunity and Forecast 2025–2030. <imarcgroup.com/isopropyl-alcohol-pricing-report> accessed 05.12.2025.

IMARC Group, 2025e, Acrylic Acid Pricing Report: Market Trends, Share, Size, Growth, Opportunity and Forecast 2025–2030. <imarcgroup.com/acrylic-acid-pricing-report> accessed 05.12.2025.

Ishaq H., Dincer I., Crawford C., 2022, A review on hydrogen production and utilization: Challenges and opportunities, International Journal of Hydrogen Energy, 47, 26238–26264.

Koranian P., Huang Q., Dalai A.K., Sammynaiken R., 2022, Chemicals production from glycerol through heterogeneous catalysis: A review, Catalysts, 12, 897.

Khor S. C., Jusoh M., Zakaria Z.Y., 2025, Hydrogen production from steam and dry reforming of methane-ethane-glycerol: A thermodynamic comparative analysis, Chemical Engineering Research and Design, 180, 178–189.

Lin Y.C., 2013, Catalytic valorization of glycerol to hydrogen and syngas, International Journal of Hydrogen Energy, 38, 2678–2700.

Methanex, 2025, About Methanol – Pricing <methanex.com/our-products/about-methanol/pricing/> accessed 05.12.2025.

Odenweller A., Ueckerdt F., 2025, The green hydrogen ambition and implementation gap, Nature Energy, 10, 110–123.

PMarket Research, 2025, Glycolic Acid Market Analysis, Trends, Growth, Share and Forecast 2025–2032 <pmarketresearch.com/glycolic-acid-market-analysis/> accessed 04.12.2025.

Rastegari H., Jazini H., Ghaziaskar H.S., Yalpani M., 2018, Applications of Biodiesel By-products, Chapter in: Tabatabaei M., Aghbashlo M. (Ed.), Biodiesel, Biofuel and Biorefinery Technologies, Vol 8, Springer, Cham, 101–125.

Sandid A., Spallina V., Esteban J., 2024, Glycerol to value-added chemicals: State of the art and advances in reaction engineering and kinetic modelling, Fuel Processing Technology, 253, 108008.

Silva C.C.G., Silva, L.H.S., Guirardello R., 2025, Synthesis of global reaction routes using thermodynamic evaluation. Chemical Engineering Transactions, 117, 865–870.

Sunsirs, 2025, The Price of Formic Acid Will Mainly Be Stable at the Beginning of 2025 <sunsirs.com/uk/detail_news-22242.html> accessed 04.12.2025.

Sunsirs, 2022, Daily Review on Ethylene Glycol <sunsirs.com/uk/detail_news-7657.html> accessed 06.12.2025.

Statista, 2025a, Global propylene production capacity from 2015 to 2024 (in million metric tons) <statista.com/statistics/1065879/global-propylene-production-capacity/> accessed 05.12.2025.

Statista, 2025b, Average price of Acrylonitrile (ACN) in China from 2019 to 2024 <statista.com/statistics/1441223/china-price-of-acrylonitrile/> accessed 05.12.2025.

Tavera Ruiz C.P., Dumeignil F., Capron M., 2021, Catalytic production of glycolic acid from glycerol oxidation: An optimization using response surface methodology, Catalysts, 11, 257.

U.S. Energy Information Administration, 2025, Weekly All Grades All Formulations Retail Gasoline Prices <eia.gov/dnav/pet/hist/LeafHandler.ashx?n=PET&s=W_EPLLPA_PWR_NUS_DPG&f=W>accessed 05.12.2025.

Vargas, B.J., de Menezes Vicenti, J.R., da Silva, E.D., Pinto, A.H., Gorup, L.F., da Rosa, C.A., Mortola, V.B., 2023, Glycerol valorization: The effect of Cu on the HZSM-5 catalyst structure and increased selectivity towards allyl alcohol. Chemical Engineering Research and Design, 196, 617–631.

Venkateshappa B., Bisarya A., Nandi P.G., Dhole S., Kumar A., 2024, Production of lactic acid via catalytic transfer dehydrogenation of glycerol catalyzed by base metal salt ferrous chloride and its NNN pincer-iron complexes, Inorganic Chemistry, 63, 15294–15310.

Xu J., Zhao Y., Xu H., Zhang H., Yu B., Hao L., Liu Z., 2014, Selective oxidation of glycerol to formic acid catalyzed by Ru(OH)₄/r-GO in the presence of FeCl₃, Applied Catalysis B: Environmental, 155, 267–273.